# CV Workbench -- Computer Vision Workbench

# Overview

The CV-Workbench is a framework that simplifies performing image processing for non-programmers. Users build scripts with multiple image processing operator steps to run different experiments with image data and store data in various formats. The advantages of the system are:

- It allows non-programmers to implement image processing. It permits sophisticated control of image operator parameters without programming.

- The system is simple. It provides an easy way to set up the input data, parameters, and outputs for image processing operations.

- It uses scripts allow reproducing results exactly, running series of operations on different sets of data, and easily changing and reusing different operations and parameters. Scripts can also be managed under computer version control systems like git.

- It is extensible using C++. A library is provided to easily programming new operations without changing the base system.

- The system is open source and free for use in educational settings or for personal use.

An experiment management module runs the image processing scripts on Linux using the command line. The module handles reading and writing images and data, and applying operators with the specified parameters. Experiment results are logged and the scripts can be re-run in a reproducible way. Test scripts are available as samples that can teach how scripts work and can be reused.

An example application may be setting up labs for an image processing course. Scripts can be provided to demonstrate different techniques, then students can modify the scripts and turn them in for assignments. This is not a full image processing system, so it may be useful for a robotics course that needs only a more limited set of image processing operations. Additional operations can be relatively easily added if C++ programming help is available.

# Method

Scripts are written as JSON text files whose format clearly defines the operators and parameters to use. Operators use and produce data in different formats, such as images, histograms, and Hough accumulators. A script records a chain of experiment steps that each specify an image processing operation with the data input and output, and parameters used by the operations. Steps can reuse data produced by previous steps. Data is stored on the filesystem, where it can be examined and reused.

The following sections describe:

- The script format, how to specify operators to use, and where on the filesystem the data is to be stored.

- The image processing operations available, with their inputs, the parameters, the outputs, and function. Links to internet sources provide more details on the image processing functions.

- The storage formats for each type of data, and log output produced by the operations.

After reading this manual, you should be able to write image processing scripts, run them, and find the outputs for visualization or further processing.

# Operators

Operators can be separated into different general types.

## Operator domains

The broadest grouping of image processing operators are domains. Sample operator domains follow.

| Domain | Description |
| --- | --- |
| Binary operators | Transformations of binary images . |
| Filters | Pixel-level operators that transform pixel values or make computations based on pixel neighborhoods . |
| Histogram operators | Creating, transforming, or extracting parameters of histograms. |
| Hough transforms | An operation to detect features of different shapes in an image. |
| Transforms | Various other image-wide changes. |

## Operator classes

Within each domain, there are operator classes for specific groups of operators. Sample operator domain classes follow.

| Domain | Operator classes |
| --- | --- |
| Binary operators | Morphological operations that shrink or grow pixel regions based on neighboring pixel values. |
| Filters | Examples are of pixel-level operators are: Edge detectors Convolutions Thresholding |
| Histogram operators | Producing frequency distributions of image pixel or other values, for example for threshold selection. |
| Hough transforms | Verifying the presence of shapes from collections of pixel values. Example operations include: Generating Hough accumulators Selecting Hough peaks corresponding to line features Overlaying line features on images |
| Transforms | These are various different image operations including: copying images and sub-images creating test images transforming pixel values to new ranges |

Specific operations are covered in section 7 below. Operator names include the domain, class and specific operation used.

## Convolution

A number of these operations produce new images based on neighborhoods of values around a pixel using convolutions. An example of the mechanics of calculating a convolution follow. The diagram shows looking at every 3x3 neighborhood of a pixel to compute a pixel value, which then is put into the output image where the center of the neighborhood is. The parameters used to calculate the new value from a neighborhood is called a kernel.  In this case, the input image is 8x8 pixels, but the pixels at the edges have no values because they do not have complete neighborhoods. In this system, the output image is 6x6 pixels with the edges are removed from the input image. The diagram gives the formula for how large the edges are that are removed.

## Histograms

Histograms are frequency distributions of image or other data values. Histograms consist of a number of bins, where each bin contains counts for a range of values. This diagram shows how histograms are calculated.

- upperin is the largest value counted in the histogram

- lowerin is the smallest value counted in the histogram

- upperout is the value counted in the highest bin of the histogram

- lowerout is the value counted in the lowest bin of the histogram

All input values higher than upperout are counted in the highest bin and all input values lower than lowerout are counted in the lowest bin.

The formula for calculating in which bin values within the range of lowerin to outerin are counted is:

bin=round# bins∙ pixelin-lowerinupperin-lowerin

The formula for the value corresponding to a particular bin is this.

value=bin∙ upperin-lowerin# bins-1+lowerin

## Hough transform

The Hough transform can convert image points to votes of where shapes can possibly occur in an image. They are used in this system to find on which line a pixel may fall. The number of points that fall on a given line can be counted to find on which lines the most points fall. The following diagram shows how the Hough transform can find image lines for each point. The polar line equation r=xcosθ + ysinθ is used to compute the polar line (r, θ) parameters that describe a line. Angles θ between 0 and 180, with a fixed increment between angles, is tested. The lines are counted and the most common ones correspond to the strongest lines in the image.

# Experiments

Each script runs an experiment that consists of a series of experiments steps. Each step consists of:

- One or more input images or other types of data

- An image processing operator

- Parameters controlling the operator function

- One or more output image or other types of data

These scripts may be run against a set of sample images to test the effectiveness of the processing. Data produced by experiments are saved on the filesystem. The processing results for operators are logged for later review.

# Data formats

Each type of data can be stored in various formats as described in section 7.2. Image data saved as unsigned 8 bit values can be visualized as a JPEG image. Much of the data is stored in signed integer for floating point format because the data values involved lie outside the unsigned 8 bit range of 0 to 255. On saving data values outside the range of 0 to 255 in unsigned 8 bit images, the exact value saved in undefined. Other data formats include tab-delimited text output that is viewable in Microsoft Excel or gnuplot, for example.

# Data Types

Image processing involves various data types. Operators can use different types of data for input and produce data of different types on output. The data types supported in the current release follow. Details on how the data types are saved on the filesystem are described in sections below.

| Data type | Description |
| --- | --- |
| Image | Two-dimensional matrices of grayscale or color values. |
| Histogram | Frequency distributions for sets of values, such as pixel values in images. |
| Hough transform accumulator data | Hough transform (rho, theta) counts for detection of lines in edge images. |
| Hough peak lines | Lines in polar coordinates selected from Hough accumulators. |

## Image data type

Images are a matrix of pixel values consists of one or more components. Grayscale images have one component value per pixel, color images have three RGB component values. Each component pixel value is stored as one of three binary formats. OpenCV “depth” codes are used to describe the binary formats.

| Value depth | Value range | Description |
| --- | --- | --- |
| CV_8U | 0..255 | unsigned byte values |
| CV_32S | INT_MIN..INT_MAX | signed 32-bit integer values |
| CV_32F | FLT_MIN..FLT_MAX | signed 32-bit floating values |

CV_8U images are stored in JPEG format. The JPEG image file format is defined by the JPEG standard. Images generated by operators generally use CV_8U, CV_32S, or CV_32F as input and generate CV_32S or CV_32F on output, except as noted in the operator descriptions. CV_32S and CV_32F images may be used interchangeably without having to specify the depth in operators, though CV_32F images are output if any input image is CV_32F. The "transform-intensity-map" operator can convert CV_32S and CV_32F images to CV_8U JPEG for display.

The binary image data file format has the following format. Color RGB pixels are stored as three consecutive pixel values, one for each component. The three pixel values represent the red, green, and blue components.

| Field | Type | Description |
| --- | --- | --- |
| rows | 32-bit integer | The number of image rows |
| cols | 32-bit integer | The number of image columns |
| components | 32-bit integer | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | 32-bit integer | Pixel value data type. CV_8U has value 0 CV_32S has value 1 CV_32F has value 2 |
| image data | rows * cols * components binary pixel values | Each value is CV_8U, CV_32S, or CV_32F as specified by the depth. Color pixels are consecutive 32-bit integer red, green, and blue values. |

## Histogram data type

Histograms count the number of different values for any data type containing a range of values, such as images or Hough accumulators. The histogram bins correspond to a data values ranging from a lower to an upper value. The number of histogram bins nbins, the lower value lower_value, and the upper value upper_value determine into which bin a value is counted as shown in section 3.4. Values below the lower value are counted in the first bin, and values above the upper value are counted in the last bin. The code to produce the bin corresponding to a value is this:

if value < lower_value

bin = 0

else if value > upper_value

bin = nbins – 1

else

bin = round((nbins – 1) * (value – lower_value) / (upper_value – lower_value))

The histogram data type format indicates how histogram data is stored. There are both binary and text stored histogram formats. Binary histogram files are used only within this software. Text histogram files may visualized with gnuplot or Microsoft Excel.

### Histogram data binary file format

Binary histogram files contain the statistics for a value distribution of the input values, such for as for an image or Hough accumulator, including statistics for the histogram such as the minimum, maximum, and total counts for all bins. The binary histogram data files have the following format:

| Field | Type | Description |
| --- | --- | --- |
| nbins | 32-bit integer | The number of histogram bins |
| lower_value | 32-bit float | The lower pixel value accumulated in the first histogram bin |
| upper_value | 32-bit float | The upper pixel value accumulated in the last histogram bin |
| histogram data | nbins * 32-bit integers | The histogram bin data |

### Histogram data text file format

Histogram text files are stored as input to display with gnuplot or import into Microsoft Excel. Two files are created. A data file output-filename.txt is generated with each histogram bin stored as a record with the format:

bin-value bin-count

where the bin-value is the value corresponding to a given bin. It has this formula, where bin no. ranges from 0 to nbins-1:

bin-value = lower_value + bin no. * (upper_value - lower_value) / (nbins – 1)

A gnuplot script file output-filename.gp is generated to display the histogram as:

set style data histograms

plot './output-filename.txt' using 2:xtic(1)

pause -1 "Hit any key to continue"

The script is run with this command, assuming gnuplot is installed:

$ gnuplot output-filename.gp

## Hough accumulator data type

Hough accumulator data contains counts from the accumulated polar line (r, θ) parameters calculated for each image point by the Hough algorithm. Lines are computed for angles  between 0 and 180 degrees, where theta is incremented by a fixed number of degrees theta_inc between samples. The number of angles nthetas samples is:

nthetas = 180 / theta_inc

For an image size (rows, cols), the polar line distance rho is computed from each (row, col) point with:

rho = (col – cols/2) * cos(theta) + (rows/2 – rows) * sin(theta)

The number of rho values nrhos for images is:

nrhos = sqrt(rows2 + columns2)

Each rho is accessed in the accumulator by bin index rho_index that corresponds to a rho with the formulas:

rho_index = rho + nrhos / 2

rho = rho_index – nrhos / 2

The Hough accumulator is indexed by (rho_index, theta_index), where theta_index is the index of the sampled angle used. theta_index corresponds to the angle:

theta degrees = theta_index * theta_inc

theta_index = theta degrees / theta_inc

Polar lines (rho, theta) are generally input and stored as (rho_index, theta_index), except as noted.

### Hough accumulator data binary file format

Binary Hough accumulator files contain the Hough accumulator counts. The binary Hough accumulator data files are defined by the workbench, and include the following fields:

| Field | Type | Description |
| --- | --- | --- |
| theta_inc | 32-bit integer | The increment between sampled angles in degrees |
| rows | 32-bit integer | The number of rows in the image used to create the accumulator |
| cols | 32-bit integer | The number of columns in the image used to create the accumulator |
| hough accumulator data | nrhos * nthetas 32-bit integers | The Hough accumulator (rho_index, theta_index) count data |

### Hough accumulator data text file format

Hough accumulator text data is stored as tab-delimited text files with the following format. These files can be imported into Microsoft Excel for review.

| first line: | rho values from -nrhos/2 to nrhos/2 |
| --- | --- |
| following lines: | sample theta in degrees, followed by each accumulator count for the corresponding (rho, theta_index) |

## Hough peak line data type

Histogram peaks are polar line (r, θ) parameters selected from the Hough accumulator for a given image. Each peak includes the accumulator count for that line.

### Hough peak line data binary file format

The binary Hough peak line data is defined by the workbench and has the following format. Polar line data is stored as (rho_index, theta_index). The corresponding rho and theta can be derived from:

rho = rho_index – max_rho / 2

theta degrees = theta_index * theta_inc

| Field | Type | Description |
| --- | --- | --- |
| nlines | 32-bit integer | The number of polar lines (rho_index, theta_index, count) |
| theta_inc | 32-bit integer | The increment between sampled angles in degrees |
| nrhos | 32-bit integer | The number of rho values sampled |
| Hough peak line data | 3 * nlines 32-bit integers | The polar line (rho_index, theta_index, count) data |

### Hough peak line data text file format

Hough peak line text data is stored as tab-delimited text files with nlines lines as a record in the following format. These files can be imported into Microsoft Excel for review.

rho-index theta_index count

# Scripts

Scripts are a series of image processing operations Scripts are written in JSON format to define the operations performed in an experiment.

## Experiments

Experiments have this JSON format:

{

"experiment": {

"steps": [

{

step-definition

},

{

step-definition

},

additional-steps

]

}

}

Each step-definition has the format:

{

"id": step-index,

operator-applied,

“input-data”: [

input-data-descriptors

],

“output-data”: [

output-data-descriptors

],

“parameters”: [

parameters-applied

]

}

Each operator-applied has the format:

"operator": "operator-name",

Multiple data descriptors may be specified for a step, for example if multiple input data sources are combined to produce the output, or if it is desired to produce both the binary and text formats for histogram data. The input-data-descriptors and output-data-descriptors are specified with:

{

"id": descriptor-index,

"type": "data-type,

"format": "data-format",

"repository": "data-storage-location",

additional-data-descriptor-parameters

}

Each parameters-supplied has this format, where all parameters including numbers have text values:

"parameter-name": "value"

## Data Descriptors

The data descriptor determines what type of data is input or output, in what format is that data stored, and in which type of repository is the data is to be found.

### Data Type

The data-type describes the type of input or output object used by an image operator as described in section 6. Image operators generally produce 32-bit signed outputs. Some image operators can read or produce 8-bit unsigned outputs that are stored in a JPEG image file. Storing images as 32-bit signed values is different from other image processing systems, but unsigned byte values cannot store the full range of values produced by image operators. They can be converted to unsigned 8-bit JPEG images for viewing with the transform-intensity-map operator, but they only approximate the true values stored in the image.

| Data type | data-type | Description |
| --- | --- | --- |
| Image | “image” | An array of pixels that are stored as integer or real numbers. Programs on most computers can view integer images with unsigned byte pixels having values from 0 to 255 stored on the filesystem in JPEG format. CV_8U indicated unsigned byte 8-bit pixels CV_32S pixels are signed 32-bit integer pixels CV_32F pixels are signed 32-bit floating point pixels |
| Histogram | “histogram” | Arrays of integers storing frequencies of different values in an object of different data types. |
| Hough accumulator data | “hough” | Counts and positions of lines in an image. |
| Hough peak lines | “polar-line” | Hough peak lines are the positions of lines in an image having a minimum number of counts that the lines occur in the image. |

### Data Format

The data-format is how an object of a data typed is stored on the filesystem. Tab-delimited text data can be displayed using Microsoft Excel or gnuplot.

| Data type | data-format |
| --- | --- |
| Image | “binary” for CV_32S or CV_32S data “jpeg” for CV_8U JPEG images “text” for tab-delimited data |
| Histogram | “binary” for binary histogram data “text” for tab-delimited histogram data |
| Hough accumulator data | “binary” for binary Hough accumulator data “text” for output tab-delimited Hough accumulator data |
| Hough peak lines | “binary” for binary Hough peak line data “text” for output tab-delimited Hough peak line data |

### Data Repository

Input and output data may be stored in different types of repositories. Currently objects are only stored in filesystem disk files, but eventually they can be stored in databases as well. Not all data types and data formats are supported for any given repository. Repositories to be supported include the following.

| Data repository | Supported | Description |
| --- | --- | --- |
| “filesystem” | yes | Files on the filesystem |
| “berkeley_db” | not yet | In Berkeley DB database records |
| “internet” | not yet | Streamed Internet data |

Each repository type has specific additional parameters.

#### Filesystem data repository

The filesystem data descriptor is specified with:

"id": descriptor-index,

"type": "data-type",

"format": "data-format",

"repository": "filesystem",

"directory": "directory-location",

"filename": "filename-base",

"ext": "filename-extension"

The filesystem data repository supports all data types and data formats listed above. The file path associated with the data descriptor is “directory/filename-base.filename-extension". The extension may default to specific values for certain operations, such as that the default extension for image JPEG data is “jpg”.

## Script run results logging

Scripts run results are saved with the original experiment run JSON text file into  a log file. Results are logged into different levels of the script. The format in which results are added to the top-level of the experiment script follows. The log results are added at the end of each step-definition in the experiment:

{

"experiment": {

“run” : {

“script-path”: “script-path”,

“run-time”: “timestamp”,

“username”: “username”,

“version”: “program-version”

},

"steps": [

{

step-definition,

results-added

},

{

step-definition,

results-added

},

additional-steps

]

experiment-run-entry

}

}

Results added to each step-definition have the format:

“results”: {

operator-specific-run-results

}

The experiment-run-entry has the format.

"run":{

"script-path":"script-json-file-path",

"run-time":"run-date-time",

"username":"user",

"version":"version"

}

In the event errors occur running operators, the step-definition will have the format:

{

"id": step-index,

operator-applied,

“input-data”: [

input-data-descriptors

],

“output-data”: [

output-data-descriptors

],

“parameters”: [

parameters-applied

],

“errors”: [

operator-run-errors

]

}

The operator-run-errors have the format:

“module-id”: “error-message”

where module is the routine in which the error occurred, id is a unique id for the error within the module, and error-message describes what error occurred. The module-id can be used to look up an explanation of the error in a separate error message document.

## Running the script

Scripts currently can be run only in Linux. The Linux program to runs scripts can be found in this directory of the Github repository:

bin/cv-workbench

A copy of this document describing that version of the program can be found at:

doc/cv-workbench manual.docx

See section 9 for details on getting files from Github. The program can be run under an Intel-based Linux distribution such as Ubuntu. A version of Linux under which the program can be run on Windows 10 that is fairly easy to install uses Windows Subsystem for Linux WSL2 with Ubuntu or Pengwin.

The program is run under Linux with this command line:

$ ./cv-workbench script-file.json

The log file is stored in script-file.log in the same directory as the script file. The script-file and log-file are as described above. Serious errors that prevent running the script, such as a missing or invalid script file, are written to the terminal.

## Sample Operator Scripts

Following are sample scripts for a Sobel operator and an image intensity map transform.

### Sobel Operator script experiment step

A sample script experiment step specifying the Sobel operator is:

{

"id": 1,

"operator": "filter-edge-sobel",

"input-data": [

{

"id": 1,

"type": "image",

"repository": "filesystem",

"format": "jpeg",

"directory": ".",

"filename": "square45-90_gray",

"ext": "jpg"

}

],

"output-data": [

{

"id": 1,

"type": "image",

"repository": "filesystem",

"format": "binary",

"directory": ".",

"filename": "square45-90_sobel_gray",

"ext": "bin"

}

],

"parameters": {

"orientation": "0"

}

}

The significant elements of the script are:

| Script segment | Description |
| --- | --- |
| { "id": 1, "operator": "filter-edge-sobel", | This is the first experiment step, id 1, for the Sobel edge operator |
| "input-data": [ { "id": 1, "type": "image", "repository": "filesystem", "format": "jpeg", "directory": ".", "filename": "square45-90_gray", "ext": "jpg" } ], | The input data for the operator is a CV_8U JPEG image file named “square45-90_gray.jpg” in the current directory |
| "output-data": [ { "id": 1, "type": "image", "repository": "filesystem", "format": "binary", "directory": ".", "filename": "square45-90_sobel_gray", "ext": "bin" } ], | The output data for the operator is a CV_32S binary image file named “square45-90_sobel_gray.bin” in the current directory. The Sobel operator defines that the image is CV_32S. |
| "parameters": { "orientation": "0" } } | The script parameter defines the Sobel orientation as 0 degrees. The mask used is: [-1, 0, 1] [-2, 0, 2] [-1, 0, 1] |

### Image Intensity Map Operator script experiment step

A sample script experiment step specifying mapping image intensities for an input binary CV_32S or CV_32F image to a CV_8U JPEG output image follows. The operator detects that the input image is in either a CV_32S or CV_32F format and handles either one.

{

"id": 1,

"operator": "transform-intensity-map",

"input-data": [

{

"id": 1,

"type": "image",

"repository": "filesystem",

"format": "binary",

"directory": ".",

"filename": "square45-90_sobel_gray",

"ext": "bin"

}

],

"output-data": [

{

"id": 1,

"type": "image",

"repository": "filesystem",

"format": "jpeg",

"directory": ".",

"filename": "square45-90_sobel_gray",

"ext": "jpg"

}

],

"parameters": {

"lower_in": "-1020",

"upper_in": "1020",

"lower_out": "0",

"upper_out": "255",

"depth": "CV_8U"

}

The significant elements of the script are:

| Script segment | Description |
| --- | --- |
| { "id": 1, "operator": "transform-intensity-map", | This is the first experiment step, id 1, for the image intensity map operator |
| "input-data": [ { "id": 1, "type": "image", "repository": "filesystem", "format": "binary", "directory": ".", "filename": "square45-90_sobel_gray", "ext": "bin" } ], | The input data for the operator is a CV_32S or CV_32F binary image file named “square45-90_gray.bin” in the current directory |
| "output-data": [ { "id": 1, "type": "image", "repository": "filesystem", "format": "jpeg", "directory": ".", "filename": "square45-90_sobel_gray", "ext": "jpg" } ], | The output data for the operator is a CV_8U binary JPEG file named “square45-90_sobel_gray.jpg” in the current directory. The CV_8U depth is implied by specifying a JPEG output format. |
| "parameters": { "lower_in": "-1020", "upper_in": "1020", "lower_out": "0", "upper_out": "255" }, | The script parameter define lower_in, upper_in, lower_out, and upper_out intensity mapping parameters. |

# Operators

Operators are predefined for various domains and classes as listed below. The current version supports only a limited set of operators, but more are added for each release. The following operators are currently defined. Those marked with ‘*’ are not yet implemented in the current version.

| Domain | Class | Instance | future | Description |
| --- | --- | --- | --- | --- |
| Filter | Edge | Gaussian | * | Difference of Gaussian edge operator |
| Filter | Edge | Kirsch |  | Kirsch edge operator |
| Filter | Edge | Laplacian | * | Laplacian of Gaussian detector |
| Filter | Edge | Prewitt |  | Prewitt edge operator |
| Filter | Edge | Roberts |  | Roberts edge operator |
| Filter | Edge | Sobel |  | Sobel edge operator |
| Filter | Image | Morphology |  | Map binary images with dilate, erode, open, close, gradient, top hat, or black at morphology transforms. |
| Filter | Smooth | Average | * | Average smoothing operator |
| Filter | Smooth | Gaussian | * | Gaussian smoothing operator |
| Filter | Smooth | Median | * | Average smoothing operator |
| Histogram | Hough | Create |  | Histogram Hough accumulator counts |
| Histogram | Image | Create |  | Histogram pixel value counts for an image |
| Histogram | Peak | Detect | * | Find histogram peaks exceeding a threshold |
| Hough | Draw | Line |  | Draw an image line segment for a polar (rho, theta) line |
| Hough | Draw | Lines | * | Draw image line segments for polar (rho, theta) lines detected by the hough-peak-detect operator |
| Hough | Image | Create |  | Accumulate Hough (rho, theta) counts from an image |
| Hough | Peak | Detect |  | Find Hough peak lines exceeding a threshold |
| Transform | Image | Copy | * | Copy image data and components from input to output images |
| Transform | Image | Create |  | Create test image from text file or from scratch optionally with superimposed features |
| Transform | Intensity | Map |  | Map CV_32S or CV_32F image intensities from an input intensity range to an output intensity range. Output images may be JPEG CV_8U images or binary CV_32S or CV_32F images. |

Sample workflows are:

| Data type | Use | Steps |
| --- | --- | --- |
| Image | Generate edges | edge filter image get image histogram and statistics analyze histogram and statistics to get edge thresholds select edges generate JPEG images for viewing |
| Hough | Detect lines | generate edge image create Hough accumulator get Hough accumulator histogram and statistics analyze histogram and statistics to get Hough line thresholds select peaks draw lines in the original JPEG image for viewing |

## filter-edge-kirsch Operator

The Kirsch edge operator convolves the image with kernels oriented to each of 8 directions as follows, where “[…], […], […]” are the rows of the kernel:

| orientation | kernel |
| --- | --- |
| N | +5+5+5-30-3-3-3-3 |
| NW | +5+5-3+50-3-3-3-3 |
| W | +5-3-3+50-3+5-3-3 |
| SW | -3-3-3+50-3+5+5-3 |
| S | -3-3-3-30-3+5+5+5 |
| SE | -3-3+5-30+5-3+5+5 |
| E | -3-3+5-30+5-3-3+5 |
| NE | -3+5+5-30+5-3-3-3 |

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “binary” or “jpeg” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameter to the Kirsch operator is:

| parameter | default | description |
| --- | --- | --- |
| orientation | required | “N”, “NW”, “W”, “SW”, “S”, “SE”, “E”, or “NE” direction as above |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output image pixel statistics:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| pixel mean | pixel value mean |
| pixel standard deviation | pixel value standard deviation |
| min pixel value | minimum pixel value |
| max pixel value | maximum pixel value |

## filter-edge-laplacian Operator

Planned but not yet implemented in the current release.

## filter-edge-prewitt Operator

The Prewitt edge operator convolves the image with kernels with orientations corresponding to 0 or 90 degrees as follows:

| orientation | kernel |
| --- | --- |
| 0 | 10-110-110-1 |
| 90 | 111000-1-1-1 |

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameter to the Prewitt operator is:

| parameter | default | description |
| --- | --- | --- |
| orientation | required | “0” or “90” degrees as above |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output image pixel statistics:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| pixel mean | pixel value mean |
| pixel standard deviation | pixel value standard deviation |
| min pixel value | minimum pixel value |
| max pixel value | maximum pixel value |

## filter-edge-roberts Operator

The Roberts cross edge operator convolves the image with kernels with orientations corresponding to 0 or 90 degrees as follows:

| orientation | kernel |
| --- | --- |
| 0 | +100-1 |
| 90 | 0+1-10 |

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameter to the Roberts operator is:

| parameter | default | description |
| --- | --- | --- |
| orientation | required | “0” or “90” degree as above |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output image pixel statistics:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| pixel mean | pixel value mean |
| pixel standard deviation | pixel value standard deviation |
| min pixel value | minimum pixel value |
| max pixel value | maximum pixel value |

## filter-edge-sobel Operator

The Sobel edge operator convolves the image with kernels with orientations corresponding to 0 or 90 degrees as follows:

| orientation | kernel |
| --- | --- |
| 0 | -10+1-20+2-10+1 |
| 90 | +1+2+1000-1-21 |

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameter to the Sobel operator is:

| parameter | default | description |
| --- | --- | --- |
| orientation | required | “0” or “90” degrees as above |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output image pixel statistics:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| pixel mean | pixel value mean |
| pixel standard deviation | pixel value standard deviation |
| min pixel value | minimum pixel value |
| max pixel value | maximum pixel value |

## filter-image-morphology Operator

The image morphology operator applies one of six different mathematical morphology operations on an image. Each operation applies a binary mask called a structuring element at each pixel position and decides as to what output value to produce. Erosion selects the minimum value under the mask and dilation selects the maximum value. This will give the appropriate result for both binary and grayscale images. The operator produce output images of the same depth as the input images.

| Operation | Compound | Description |
| --- | --- | --- |
| Erosion |  | Output the minimum value under the structuring element |
| Dilation |  | Output the maximum value under the structuring element |
| Opening | x | Erosion followed by dilation |
| Closing | x | Dilation followed by eroding |
| Gradient | x | Subtract the eroded image from the dilated image |
| Top Hat | x | Subtract the opened image from the original image |
| Black Hat | x | Subtract the original image from the closed image |

The operator takes the following data descriptors. The output data format is ignored.

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameters to the Morphology operator are:

| parameter | default | description |
| --- | --- | --- |
| structuring-element | required | Shape of the structuring element: “rectangle”, “cross”, “ellipse” |
| height | required | Rectangle, ellipse, or cross pixel height |
| width | required | Rectangle, ellipse, or cross pixel width |
| thickness | required for “cross” | Cross pixel thickness |
| operation | required | “erode”, “dilate”, “open”, “close”, “gradient”, “top-hat”, or “black-hat” |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output image pixel parameters:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| mean | pixel value mean |
| standard deviation | pixel value standard deviation |
| min value | minimum pixel value |
| max value | maximum pixel value |

## filter-smooth-average Operator

The filter smooth average operator convolves the image with a kernel that averages the pixels under the kernel. For a kernel that is size rows x cols, the output pixel is the sum of the pixels in a rectangle rows x cols around the input pixel, divided by 1/(rows * cols).

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameters to the smooth average operator are:

| parameter | default | description |
| --- | --- | --- |
| rows | required | kernel height |
| cols | required | kernel width |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output image pixel statistics:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| pixel mean | pixel value mean |
| pixel standard deviation | pixel value standard deviation |
| min pixel value | minimum pixel value |
| max pixel value | maximum pixel value |

## filter-smooth-gaussian Operator

The filter smooth gaussian operator convolves the image with a gaussian kernel that does a weighted average of the pixels under the kernel. The 1-dimensional gaussian using variance  σx can be computed by convolving with this this formula:

Gx=1σx2πe-x22σx2

The gaussian has a separable kernel, so the 2-dimensional gaussian can be computed asymmetrically by convolving with the above 1-dimensional formula in the x direction and with the following formula using variance  σx in the y direction:

Gy=1σy2πe-y22σy2

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameters to the smooth average operator are:

| parameter | default | description |
| --- | --- | --- |
| rows | required | kernel height |
| cols | required | kernel width |
| sigma-x | required | column variance |
| sigma-y | required | row variance |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output image pixel statistics:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| pixel mean | pixel value mean |
| pixel standard deviation | pixel value standard deviation |
| min pixel value | minimum pixel value |
| max pixel value | maximum pixel value |

## filter-smooth-median Operator

Planned but not yet implemented in the current release.

## histogram-hough-create Operator

The create Hough accumulator histogram operator creates a histogram for a Hough accumulator and stores it in binary or text format.

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “hough” | “binary” | input Hough accumulator |
| output | 1 | “histogram” | “binary” or “text” | output Hough accumulator histogram |

For text output format, the output data descriptor extension is ignored. The output text file has the extension “.txt” and the output gnuplot script has the extension “.gp” as described in section 5.2.2.

The parameters to the create hough histogram operator are:

| parameter | default | description |
| --- | --- | --- |
| nbins | required | The number of histogram bins |
| lower-value | required | The minimum value accumulated in the first histogram bin |
| upper-value | required | The maximum value accumulated in the last histogram bin |

The experiment log output includes the histogram and Hough accumulator statistics:

| Field | Description |
| --- | --- |
| lower_value | The minimum value accumulated in the first histogram bin |
| upper_value | The maximum value accumulated in the last histogram bin |
| accumulator count | accumulator nrhos * nthetas |
| accumulator mean | accumulator value mean |
| accumulator standard deviation | accumulator value standard deviation |
| min accumulator value | minimum accumulator value |
| max accumulator value | maximum accumulator value |
| min bin count | The lowest count for any histogram bin |
| max bin count | The largest count for any histogram bin |

## histogram-image-create Operator

The create image histogram operator creates a histogram for an image and stores it in binary or text format.

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “histogram” | “binary” or “text” | output image histogram |

For text output format, the output data descriptor extension is ignored. The output text file has the extension “.txt” and the output gnuplot script has the extension “.gp” as described in section 7.2.2.

The parameters to the create image histogram operator are. The lower_value and upper_value parameters may be omitted and default to the image minimum and maximum values.

| parameter | default | description |
| --- | --- | --- |
| nbins | required | The number of histogram bins |
| lower-value | required | The minimum value accumulated in the first histogram bin |
| upper-value | required | The maximum value accumulated in the last histogram bin |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the histogram and image statistics.

| Field | Description |
| --- | --- |
| lower_value | The minimum value accumulated in the first histogram bin |
| upper_value | The maximum value accumulated in the last histogram bin |
| pixel count | image rows * cols * components |
| pixel mean | pixel value mean |
| pixel standard deviation | pixel value standard deviation |
| min pixel value | minimum pixel value |
| max pixel value | maximum pixel value |
| min bin count | The lowest count for any histogram bin |
| max bin count | The largest count for any histogram bin |

## hough-draw-line Operator

The Hough draw line operator draws a line segment corresponding to a polar line (rho, theta) in an image and stores the resulting image. JPEG input images can be stored as either JPEG or binary output images, but binary input images can only be stored as binary output images. Grayscale input images produce grayscale output images, and color input images produce color output images. Lines are drawn in the color output image out_component component if specified. The output color image component defaults to 1 if omitted and is ignored for grayscale images.

The operator takes the following data descriptors. The output data format is ignored.

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameters to the draw Hough draw line operator are:

| parameter | default | description |
| --- | --- | --- |
| theta-inc | required | increment for the Hough accumulator angle. The number of angles are 180 / theta_inc. |
| rho | required | Polar line rho |
| theta-index | required | Polar line angle |
| pixel-value | required | Pixel value for drawn line segment |
| out-component | 1 | Color output image component in which to draw line |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output is the line segment (start-row, start-col) and (end-row, end-col) data:

| Field | Description |
| --- | --- |
| start-row | line segment start point row |
| start-col | line segment start point column |
| end-row | line segment end point row |
| end-col | line segment end point column |

## hough-draw-lines Operator

Planned but not yet implemented in the current release.

## hough-image-create Operator

The create image Hough operator creates a Hough accumulator from an image and stores it in binary or text format as described in section 7.3.

The operator takes the following data descriptors.

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg” or “binary” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “hough” | “binary” or “text” | output Hough accumulator data |

The parameters to the create image Hough operator is:

| parameter | default | description |
| --- | --- | --- |
| theta-inc | 3 | increment for the Hough accumulator angle. The number of angles are 180 / theta_inc. |
| pixel-threshold | required | pixel value threshold above which image points are candidates for accumulating |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output Hough accumulator statistics:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| theta_inc | The increment between sampled angles in degrees |
| nthetas | The number of angles sampled = 180 / theta_inc |
| nrhos | The number of rho values sampled |
| bin min_count | The lowest count for any histogram bin |
| bin max_count | The largest count for any histogram bin |
| bin mean | The mean for the hough accumulator counts |
| bin standard deviation | The standard deviation for the hough accumulator counts |

## hough-peak-detect Operator

The Hough peak detect operator finds polar lines (rho, theta) in the Hough accumulator data where the counts exceed a threshold..

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “hough” | “binary” | input Hough accumulator data |
| output | 1 | “peak” | “binary” or “text” | output Hough accumulator polar line peaks. |

The parameters to the draw Hough peak detect operator are as follows. Only one of either the threshold or max_peaks parameter should be supplied.

| parameter | description |
| --- | --- |
| max-peaks | The maximum number of peaks to return |

The experiment log output is the peak polar line (rho, theta) data:

| Field | Description |
| --- | --- |
| npeaks | Number of peaks returned |
| nthetas | The number of angles sampled = 180 / theta_inc |
| nrhos | The number of rho values sampled |

## transform-image-combine Operator

The transform image combine operator does a linear recombination of two images. The formula for the output values are:

output(row, col) = scale1 * image1(row, col) + scale2 * image2(row, col) + offset

The operator takes the following data descriptors. Input images must be the same (rows, cols) size.

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg”, “binary”, “text” | input JPEG CV_8U, binary CV_32S/F, or text CV_32S intensity value file. |
| input | 2 | “image” | “jpeg”, “binary”, “text” | input JPEG CV_8U, binary CV_32S/F, or text CV_32S intensity value file. |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

These are the parameters to the draw transform image create operator.

| parameter | default | description |
| --- | --- | --- |
| scale-1 | 1.0 | Scale factor for first image value |
| scale-2 | 1.0 | Scale factor for second image value |
| offset | 0.0 | Offset for output image value |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The experiment log output includes the output image pixel parameters:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |

## transform-image-copy Operator

The transform image copy operator copies image data and components from input to output images. If the input image data descriptor is omitted, an empty output image is created. The function of the operator depends on whether input and output images are grayscale or color.

| Input Image | Output Image | Description |
| --- | --- | --- |
| Grayscale | Grayscale | Copy the input image to the output image. |
| Grayscale | Color | Copy the input image to the output image out_component component. Copy the input image to the first output image component if the out_component parameter is omitted. |
| Color | Grayscale | Copy the input image in_component component to the output image in_component component. Copy the first input image component to the output image if the in_component parameter is omitted. |
| Color | Color | Copy from the input image to the output image as below. |

There are several variations depending whether the in_component or out_component parameters are specified.

| in_component | out_component | description |
| --- | --- | --- |
| specified | specified | Copy the input image in_component to the output image out_component component |
| specified | omitted | Copy the input image in_component component to the in_component component of the output image |
| omitted | specified | Copy the input image out_component component to the output image out_component component |
| omitted | omitted | Copy all input image components to the output image |

The parameters to the draw transform image copy operator are:

| parameter | description |
| --- | --- |
| in-component | The component of the input image to copy |
| out-component | The component of the output image to copy from the input image |

The experiment log output includes the output image pixel parameters:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| mean | pixel value mean |
| standard deviation | pixel value standard deviation |
| min value | minimum pixel value |
| max value | maximum pixel value |

## transform-image-create Operator

The transform image create operator creates an image from text file or from scratch, optionally with superimposed features. This is useful to create test images. The operator takes the following data descriptors. Various parameters optionally draw points, lines, or rectangles in the image.

- If the input descriptor is omitted, an image is created from scratch and the ‘rows’ and ‘cols’ parameters are required. The output image is CV_32S data format.

- If a text input file is specified, each line consists of space or tab delimited numeric intensities. The number of rows is the number of lines in the file. The number of columns is the number of fields on each line. The number of fields must be the same on each line. If the output format is JPEG, field values outside the range of 0..255 will be truncated.

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “jpeg”, “binary”, “text”, or omitted | input JPEG CV_8U, binary CV_32S/F, or text CV_32S intensity value file. |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

These are the parameters to the draw transform image create operator. The features will be added with the foreground value. The background parameter is not yet used.

| parameter | default | description |
| --- | --- | --- |
| rows | required | The number of image rows for a new image. Required if a text input image is omitted, else ignored. |
| cols | required | The number of image columns for a new image. Required if a text input image is omitted, else ignored. |
| background | 0.0 | The base pixel value of the output image, defaults to 0 |
| foreground | 255.0 | The pixel value of added features the output image, defaults to 255 |
| point | optional | A point to add to the image as ‘row,col’ |
| line | optional | A line segment ‘(row,col):(row,col)’ to add to the image, where each ‘(row,col)’ is the start and end points |
| rectangle | optional | A rectangle ‘(row,col):(row,col)’ to add to the image, where the first ‘row,col’ is the upper left-hand corner and the second ‘row,col’ is the lower right-hand corner |
| rectangle-filled | optional | Similar to the ‘rectangle’ parameter, but creates a filled rectangle. |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

The ‘point’, ‘line’, ‘rectangle’, and ‘rectangle-filled’ parameter values may be repeated by separating the values with ‘|’, for example:

“point”: “(row,col)|(row,col)”

“line”: “(row,col):(row,col)|(row,col):(row,col)”

“rectangle”: “(row,col):(row,col)|(row,col):(row,col)”

The experiment log output includes the output image pixel parameters:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |

## transform-intensity-map Operator

The transform intensity map operator maps the input image pixel value range to the specified output image pixel value range. Input JPEG and binary images can map to CV_8U JPEG images or CV_32S and CV_32F binary images The mapping of input to output intensity values are determined as follows.

if input value < lower_in

output value = lower_out

else if input value >= upper_in

output value = upper_out

else

output value =

lower_out + (input value - lower_in) * (upper_out - upper_in) / (lower_out - lower_in)

The operator can threshold images by specifying lower_in = upper_in = threshold value. Lower_out then is the output pixel value for input pixel values < threshold, and upper_out is the output pixel value for input pixel values >= threshold. Values for JPEG output images will be CV_8U and truncated to 0..255.

The operator takes the following data descriptors:

| Descriptor type | Id | type | Format | Description |
| --- | --- | --- | --- | --- |
| input | 1 | “image” | “binary”, “jpeg”, or “text” | input CV_8U, CV_32S, or CV_32F intensity image |
| output | 1 | “image” | “binary”, “jpeg”, or “text” | output CV_8U, CV_32S, or CV_32F edge image |

The parameters to the draw transform intensity map operator are:

| parameter | default | description |
| --- | --- | --- |
| depth | required | The output image depth: CV_8U, CV_32S, or CV_32F. Defaults to the input image depth, ignored for JPEG output images. |
| lower-in | required | The lower mapped pixel value of the input image |
| upper-in | required | The lower mapped pixel value of the input image |
| lower-out | required | The lower mapped pixel value of the output image |
| upper-out | required | The upper mapped pixel value of the output image |
| min-row | 0 | minimum sub-image row |
| max-row | image rows - 1 | maximum sub-image row |
| min-col | 0 | minimum sub-image column |
| max-col | image cols - 1 | maximum sub-image column |

If lower_in or lower_out are omitted, they default to the lowest input image pixel value or highest input image pixel value.

The experiment log output includes the output image pixel parameters:

| Field | Description |
| --- | --- |
| rows | The number of image rows |
| cols | The number of image columns |
| components | The number of image components. Grayscale images have 1 component, color images have 3 components. |
| depth | Pixel value data type. CV_8U CV_32S CV_32F |
| pixel count | image rows * cols * components |
| mean | pixel value mean |
| standard deviation | pixel value standard deviation |
| min value | minimum pixel value |
| max value | maximum pixel value |

# Github Repository

You can find the CV-workbench files in Github at this link:

https://github.com/kushnertodd/cv-workbench

Following are files you can get from Github to run CV-workbench.

## This documentation

A copy of this document describing that version of the program can be found at:

doc/cv-workbench manual.docx

## CV-workbench overview and sample demonstration

This file gives a brief overview of and instructions for running a CV-workbench sample application.

doc/demonstration/README.txt

## Main CV-workbench module

The Linux program to runs scripts can be found in this directory of the Github repository:

bin/cv-workbench

## CV-workbench programs

There are a number of CV-workbench programs included in the bin/ directory of the Github repository. Many are just used as test programs. These are described below.

cv-workbench

image-copy

image-dump

image-info

image-map

image-morph

image-read

image-trig

jpegtran

### cv-workbench

This is the main CV-workbench program. It is run with:

$ ./cv-workbench script-file.json

The script file format is described above, and the log file is written to the same directory as the script file.

### image-copy

This is a program to copy a sub-image of an image to a new file. It is run with:

$./image-copy in_filename out-filename min-row min_col max_row max_col

where the parameters are:

| parameter | description |
| --- | --- |
| min-row | minimum sub-image row |
| max-row | maximum sub-image row |
| min-col | minimum sub-image column |
| max-col | maximum sub-image column |

### image-dump

This is a program to write out the contents of an image as a tab-delimited text file. It is run with:

$ ./image-dump in-filename.ext

The extension ext of the image in-filename has a “.jpg” or “.bin” extension to indicate whether it is a JPEG image file or a binary internal format image file. It is written in tab-delimited form to a file named in-filename.txt in the same directory as in-filename.

### image-info

This is a program to print statistics of an image file. It is run with:

$ ./image-info in-filename.ext

The extension ext of the image in-filename has a “.jpg” or “.bin” extension to indicate whether it is a JPEG image file or a binary internal format image file. It writes statistics of the image similar to this:

$ ./image-info test/reference/du_map_gray.jpg

filename test/reference/du_map_gray.jpg

rows                556

cols                685

components          1

depth               CV_8U

row_stride          685

npixels             380860

histogram:

nbins               100

lower_value         31

upper_value         255

bounds:

min_value           0

max_value           24757

stats:

count               380860

mean                148.429

variance            1126.51

sample variance     1126.51

standard deviation  33.5635

min value           31

max value           255

### image-map

This is a program that performs the same function as the transform-intensity-map operator. It is run with:

$ ./image-map in_filename out-filename lower-in upper-in lower-out upper-out

See Section 8.19 on the transform-intensity-map operator for a description of the parameters.

### image-morph

This is a program that tests the filter-image-morphology operator. It is runs all morphology operators on the input image with:

$ ./image-morph in_filename rows cols thickness

See Section 8.6 for a description of the filter-image-morphology operator.

### image-read

This is a program that reads a tab-delimited text image file and writes it out as an internal binary format file. It is run with:

$ ./image-read in-filename.txt

It writes the tab-delimited image file to a binary image file named in-filename.bin in the same directory as in-filename.

### image-trig

This program writes out the static cosine and sine data tables used in the file polar_trig_static.cpp. It is run with:

./image-trig

### jpegtran

This program converts color JPEG image files to grayscale files. It is run with:

$ ./jpegtran in-filename.jpg >out-filename.jpg

The file in-filename.jpg is a 3-component JPEG color image and out-filename.jpg is a 1-component JPEG grayscale image.

### Installing CV Workbench from source

To develop new operators in C++, you can follow the instructions in this file to set up your system to run and optionally build CV Workbench:

https://github.com/kushnertodd/cv-workbench/blob/master/INSTALL.md

Linux can be easily installed on Windows using WSL. Apple MacOS already comes with Linux used in Terminal windows. You can manage the image files using Windows or MacOS, but you do have to learn a little Linux. What you need to learn isn’t hard.